# MosUsbCe

# USB to Ethernet Device Driver

# For Windows CE .NET

MosChip Semiconductor Technology Ltd

Version 0.1, April 20, 2005

- Contents

1. Introduction	1

2. Resource Requirements	1

3. Install the Driver	1

4. Configure the IP Address	2

## 1. Introduction

- This document provides the information about how to install the driver for MOSCHIP MCS 7830 USB2MAC device on windows CE .NET 4.2 environments using the CAB file.

## 2. Resource Requirements

- For installation of the driver using .cab file, the following resources are needed:

- Windows CE Embedded Device with Windows CE 4.2 OS

- PC having Win 2k/XP with Microsoft ActiveSync.

- NULL Modem Cable (internally connected strictly according to RS232 standard)

- MCS 7830 USB2MAC device.

## 3. Install the Driver

The Installation of the MosUsbCe Driver by a .cab file can be done through the following steps:

- In the PC, Microsoft ActiveSync should be installed.

- Connect the WinCE Device (Embedded CE device) and the PC through a Null modem cable.

- Run ActiveSync in the PC.

- Open the Command Prompt in WinCE Device and type the command ‘repllog’ and press Enter.

- A New Partnership window appears in ActiveSync. Select No and then click on Next.

- Then it will establish a connection between PC and WinCe Device and it will show the connection status as Connected on ActiveSync.

- Then click on the Explore button on the ActiveSync window, and copy the MosUsbCe WinZip (it is actually a CAB file) file to the Mobile Device folder on your PC.

- Then open My Computer (or Corresponding folder) in the WinCe Device (Embedded CE device) and the CAB file (MosusbCe) will be there.

- To install the driver, just double click on the CAB file (MosUsbCe).

- An installation window will appear. Click OK to install. The driver is now installed.

- Now, insert the MCS 7830 USB2MAC device in to USB port on the WinCE Device. The driver will be loaded and the device is ready for connection to other machines.

## 4. Configure the IP Address

After the installation of the driver, configure the IP Address of the device as follows:

Note: The following procedure is for CEPC platform (x86). The following procedure might not be the same for Embedded CE device.

- After plugging the device on the USB port, click on the Start button.

- Go to Settings and click on the Network and Dial-up Connections

- Check whether any instance of the device (e.g. MOSUSBCE1) on the Network and Dial-up Connections. If it is there, the device is installed.

- Double click on the instance of the device.

- In the Settings window, give the IP address (e.g. 192.168.0.1 ) and Subnet Mask (e.g. 255.255.255.0) and click on OK

- After a few seconds, IP Address should be configured. Then check the IP Address on the Command Prompt by using ipconfig

Note: If you don’t configure the IP Address, it will configure a default IP Address