# Mosusbce

# USB to Ethernet Device Driver

# For Windows CE 5.0

MosChip Semiconductor Technology Ltd

Version 1.13, August 7, 2009

- Contents

For Windows CE 5.0	1

1. Introduction	1

2. File Description	1

3. Driver Installation	2

4. Configure the IP Address	3

## 1. Introduction

- This document provides the information about how to install the driver for MOSCHIP MCS 7830 USB2MAC device on windows CE 5.0 environment using the CAB file or integrating it in NK .bin image.

## 2. File Description

- Moschip driver package comes with these files:

- mosusbce.dll                   - driver file

- mosusbce.cab                  - driver cab file

- mosusbce.bib	          - sample bib file

mosusbce.reg                  - sample reg file

## 3. Driver Installation

The Installation of the MosUsbCe Driver can be done in three ways:

Customer can use any one that he/she find applicable.

- Take a flash/pen drive copy the “mosusbce.cab” file provided and install on required platform, MCS7830 is installed and ready to use on required platform.

- If you can use platform builder tools on required platform, make use of “Remote File Viewer” tool to import cab file on to platform and install it on device.

- Please follow the below procedure if you want to integrate “mosusbce.dll” in “NK.bin” image.

Procedure to integrate MCS7830 driver into target platform:

Note: Make the “OSDesign” first with Zero Errors, then add the following changes to your platform.

1) Search file “platform. Bib” in BSP provided, open, and add the following lines to the file and save it:

MODULES

;           Name            Path                                                          Memory Type

;  --------------  ---------------------------------------------  --------------------------------------

mosusbce.dll      $(_FLATRELEASEDIR)\mosusbce.dll               NK       SH

2) Search “platform.reg” file in BSP, open, add the following lines to it and save it:

; ******** Registry values for the USB to Ethernet Driver *****

[HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30768\255_0_255\Default\mosusbce]

"Dll"="mosusbce.DLL"

[HKEY_LOCAL_MACHINE\Drivers\USB\ClientDrivers\Default]

"Dll"="mosusbce.DLL"

3) Copy and Paste the “mosusbce.dll” provided into “RelDir” of build “OSDesign”

(Drive :> \WINCE500\PBWorkspaces\“OSDesign Name” \RelDir\CEPC_x86_Release)

Here in example we paste the mosusbce.dll into “OSDesign” for Release type on CEPC device.

4) Build the platform again as you did earlier, ensure zero errors.

5) From platform builder select “Build OS” -> “Make Run Time Image”.

6) Ensure zero errors; MCS7830 driver is ready for use on required platform.

Note: If you get error that “mosusbce.dll” file is not on disk, and then run these steps:

1) Step3 and   Step5.

## 4. Configure the IP Address

After the installation of the driver, configure the IP Address of the device as follows:

Note: The following procedure is for CEPC platform (x86).

- After plugging the device on the USB port, click on the Start button.

- Go to Settings and click on the Network and Dial-up Connections

- Check whether any instance of the device (e.g. MOSUSBCE1) on the Network and Dial-up Connections. If it is there, the device is installed.

- Double click on the instance of the device.

- In the Settings window, give the IP address (e.g. 192.168.xx.xx ) and Subnet Mask (e.g. 255.255.255.0) and click on OK

- After a few seconds, IP Address should be configured. Then check the IP Address on the Command Prompt by using ipconfig

Note: If you don’t configure the IP Address, it will configure a default IP Address